SER02:/home/emin#

SER02:/home/emin#

SER02:/home/emin# openser

Listening on

udp: 192.168.1.15 [192.168.1.15]:5060

tcp: 192.168.1.15 [192.168.1.15]:5060

Aliases:

tcp: SER02:5060

udp: SER02:5060

WARNING: no fork mode

0(18096) init_tcp: using epoll_lt as the io watch method (auto detected)

0(0) INFO: statistics manager successfully initialized

0(0) StateLess module - initializing

0(0) TM - initializing...

0(0) Maxfwd module- initializing

0(0) INFO:ul_init_locks: locks array size 512

0(0) TextOPS - initializing

0(0) AVPops - initializing

0(0) INFO: udp_init: SO_RCVBUF is initially 109568

0(0) INFO: udp_init: SO_RCVBUF is finally 219136

0(18096) INFO:mi_fifo:mi_child_init(1): extra fifo listener processes created

0(18096) [ New Transaction ]

0(18096) [ New Transaction ]

0(18096) [ New Transaction ]

0(18096) [  Method REGISTER from 192.168.1.11:5060  ]

REGISTER sip:192.168.1.15 SIP/2.0

Via: SIP/2.0/UDP 192.168.1.11;branch=z9hG4bK13b821e7358899c7

From: <sip:309@192.168.1.15>;tag=d5c5b42370fbe61f

To: <sip:309@192.168.1.15>

Contact: <sip:309@192.168.1.11>

Supported: replaces

Call-ID: f68433fd4c7dd243@192.168.1.11

CSeq: 101 REGISTER

Expires: 150

User-Agent: Grandstream BT110 1.0.8.33

Max-Forwards: 70

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Content-Length: 0

0(18096) [  End of Request (new transaction 1)  ]

0(18096) Location is saved; now exiting

0(18096) [ New Transaction ]

0(18096) [ New Transaction ]

0(18096) [ New Transaction ]

0(18096) [  Method REGISTER from 192.168.1.10:5060  ]

REGISTER sip:192.168.1.15 SIP/2.0

Via: SIP/2.0/UDP 192.168.1.10;branch=z9hG4bK3fe12f7c7c55fa6c

From: <sip:308@192.168.1.15>;tag=852d8ad48338a2c4

To: <sip:308@192.168.1.15>

Contact: <sip:308@192.168.1.10>

Supported: replaces

Call-ID: 1423aa41f7709130@192.168.1.10

CSeq: 144 REGISTER

Expires: 150

User-Agent: Grandstream BT110 1.0.8.33

Max-Forwards: 70

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Content-Length: 0

0(18096) [  End of Request (new transaction 2)  ]

0(18096) Location is saved; now exiting

0(18096) [ New Transaction ]

0(18096) [ New Transaction ]

0(18096) [ New Transaction ]

0(18096) [  Method INVITE from 192.168.1.10:5060  ]

INVITE sip:309@192.168.1.15 SIP/2.0

Via: SIP/2.0/UDP 192.168.1.10;branch=z9hG4bK94bea4ca587ee4aa

From: <sip:308@192.168.1.15>;tag=98d29722f6c1e57a

To: <sip:309@192.168.1.15>

Contact: <sip:308@192.168.1.10>

Supported: replaces

Call-ID: 80eff760a269fde9@192.168.1.10

CSeq: 13768 INVITE

User-Agent: Grandstream BT110 1.0.8.33

Max-Forwards: 70

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Content-Type: application/sdp

Content-Length: 225

v=0

o=308 8000 8000 IN IP4 192.168.1.10

s=SIP Call

c=IN IP4 192.168.1.10

t=0 0

m=audio 5004 RTP/AVP 4 18 97

a=sendrecv

a=rtpmap:4 G723/8000

a=rtpmap:18 G729/8000

a=rtpmap:97 iLBC/8000

a=fmtp:97 mode=20

a=ptime:60

{end}

0(18096) [  End of Request (new transaction 3)  ]

0(18096) Adding Record-Route headers

0(18096) Message is relayed; now exiting

0(18096)

[  Reply 100 (Trying) from 192.168.1.11:5060 concerning INVITE  ]

SIP/2.0 100 Trying

Via: SIP/2.0/UDP 192.168.1.15;branch=z9hG4bK2601.4ab8f3e4.0

Via: SIP/2.0/UDP 192.168.1.10;branch=z9hG4bK94bea4ca587ee4aa

From: <sip:308@192.168.1.15>;tag=98d29722f6c1e57a

To: <sip:309@192.168.1.15>

Call-ID: 80eff760a269fde9@192.168.1.10

CSeq: 13768 INVITE

User-Agent: Grandstream BT110 1.0.8.33

Content-Length: 0

[  End of Reply (transaction 3)  ]

0(18096)

[  Reply 180 (Ringing) from 192.168.1.11:5060 concerning INVITE  ]

SIP/2.0 180 Ringing

Via: SIP/2.0/UDP 192.168.1.15;branch=z9hG4bK2601.4ab8f3e4.0

Via: SIP/2.0/UDP 192.168.1.10;branch=z9hG4bK94bea4ca587ee4aa

Record-Route: <sip:192.168.1.15;lr=on;ftag=98d29722f6c1e57a>

From: <sip:308@192.168.1.15>;tag=98d29722f6c1e57a

To: <sip:309@192.168.1.15>;tag=8345c68ebf2e5dc3

Call-ID: 80eff760a269fde9@192.168.1.10

CSeq: 13768 INVITE

User-Agent: Grandstream BT110 1.0.8.33

Content-Length: 0

[  End of Reply (transaction 3)  ]

0(18096) [  Method CANCEL from 192.168.1.10:5060  ]

CANCEL sip:309@192.168.1.15 SIP/2.0

Via: SIP/2.0/UDP 192.168.1.10;branch=z9hG4bK94bea4ca587ee4aa

From: <sip:308@192.168.1.15>;tag=98d29722f6c1e57a

To: <sip:309@192.168.1.15>

Supported: replaces

Call-ID: 80eff760a269fde9@192.168.1.10

CSeq: 13768 CANCEL

User-Agent: Grandstream BT110 1.0.8.33

Max-Forwards: 70

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Content-Length: 0

0(18096) [  End of Request (transaction 0)  ]

0(18096) Message is relayed; now exiting

0(18096)

[  Reply 200 (OK) from 192.168.1.11:5060 concerning CANCEL  ]

SIP/2.0 200 OK

Via: SIP/2.0/UDP 192.168.1.15;branch=z9hG4bK2601.4ab8f3e4.0

Via: SIP/2.0/UDP 192.168.1.10;branch=z9hG4bK94bea4ca587ee4aa

From: <sip:308@192.168.1.15>;tag=98d29722f6c1e57a

To: <sip:309@192.168.1.15>;tag=8345c68ebf2e5dc3

Call-ID: 80eff760a269fde9@192.168.1.10

CSeq: 13768 CANCEL

User-Agent: Grandstream BT110 1.0.8.33

Contact: <sip:309@192.168.1.11>

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Supported: replaces

Content-Length: 0

[  End of Reply (transaction 0)  ]

0(18096)

[  Reply 487 (Request Cancelled) from 192.168.1.11:5060 concerning INVITE  ]

SIP/2.0 487 Request Cancelled

Via: SIP/2.0/UDP 192.168.1.15;branch=z9hG4bK2601.4ab8f3e4.0

Via: SIP/2.0/UDP 192.168.1.10;branch=z9hG4bK94bea4ca587ee4aa

Record-Route: <sip:192.168.1.15;lr=on;ftag=98d29722f6c1e57a>

From: <sip:308@192.168.1.15>;tag=98d29722f6c1e57a

To: <sip:309@192.168.1.15>;tag=8345c68ebf2e5dc3

Call-ID: 80eff760a269fde9@192.168.1.10

CSeq: 13768 INVITE

User-Agent: Grandstream BT110 1.0.8.33

Content-Length: 0

[  End of Reply (transaction 3)  ]

0(18096) [  Method ACK from 192.168.1.10:5060  ]

ACK sip:309@192.168.1.15 SIP/2.0

Via: SIP/2.0/UDP 192.168.1.10;branch=z9hG4bK94bea4ca587ee4aa

From: <sip:308@192.168.1.15>;tag=98d29722f6c1e57a

To: <sip:309@192.168.1.15>;tag=8345c68ebf2e5dc3

Contact: <sip:308@192.168.1.10>

Call-ID: 80eff760a269fde9@192.168.1.10

CSeq: 13768 ACK

User-Agent: Grandstream BT110 1.0.8.33

Max-Forwards: 70

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Content-Length: 0

0(18096) [  End of Request (transaction 0)  ]

0(18096) Method is ACK

0(18096) Message is relayed; now exiting

1(18097) INFO: signal 2 received

0(18096) child process 18097 exited normally, status=0

0(18096) INFO: dont_fork turned on, living on

0(18096) INFO: signal 2 received

0(18096) INFO:mi_fifo:mi_destroy: seems that fifo child is already dead!

0(18096) Thank you for flying openser

SER02:/home/emin#

SER02:/home/emin#

SER02:/home/emin#