SER02:/home/emin#

SER02:/home/emin#

SER02:/home/emin# openser

Listening on

udp: 192.168.1.15 [192.168.1.15]:5060

tcp: 192.168.1.15 [192.168.1.15]:5060

Aliases:

tcp: SER02:5060

udp: SER02:5060

WARNING: no fork mode

0(18983) init_tcp: using epoll_lt as the io watch method (auto detected)

0(0) INFO: statistics manager successfully initialized

0(0) StateLess module - initializing

0(0) TM - initializing...

0(0) Maxfwd module- initializing

0(0) INFO:ul_init_locks: locks array size 512

0(0) TextOPS - initializing

0(0) AVPops - initializing

0(0) INFO: udp_init: SO_RCVBUF is initially 109568

0(0) INFO: udp_init: SO_RCVBUF is finally 219136

0(18983) INFO:mi_fifo:mi_child_init(1): extra fifo listener processes created

0(18983) [ New Transaction ]

0(18983) [ New Transaction ]

0(18983) [ New Transaction ]

0(18983) [  Method REGISTER from 192.168.1.10:5060  ]

REGISTER sip:192.168.1.15 SIP/2.0

Via: SIP/2.0/UDP 192.168.1.10;branch=z9hG4bKf8f8a6d76cf85334

From: <sip:308@192.168.1.15>;tag=7c8757041cbd1cff

To: <sip:308@192.168.1.15>

Contact: <sip:308@192.168.1.10>

Supported: replaces

Call-ID: d628af7c33c19316@192.168.1.10

CSeq: 100 REGISTER

Expires: 150

User-Agent: Grandstream BT110 1.0.8.33

Max-Forwards: 70

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Content-Length: 0

0(18983) [  End of Request (new transaction 1)  ]

0(18983) Location is saved; now exiting

0(18983) [ New Transaction ]

0(18983) [ New Transaction ]

0(18983) [ New Transaction ]

0(18983) [  Method REGISTER from 192.168.1.11:5060  ]

REGISTER sip:192.168.1.15 SIP/2.0

Via: SIP/2.0/UDP 192.168.1.11;branch=z9hG4bK43b841e75588a9c7

From: <sip:309@192.168.1.15>;tag=d5c5b42370fbe61f

To: <sip:309@192.168.1.15>

Contact: <sip:309@192.168.1.11>

Supported: replaces

Call-ID: 078443fd5c7dd243@192.168.1.11

CSeq: 102 REGISTER

Expires: 150

User-Agent: Grandstream BT110 1.0.8.33

Max-Forwards: 70

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Content-Length: 0

0(18983) [  End of Request (new transaction 2)  ]

0(18983) Location is saved; now exiting

0(18983) [ New Transaction ]

0(18983) [ New Transaction ]

0(18983) [ New Transaction ]

0(18983) [  Method INVITE from 192.168.1.10:5060  ]

INVITE sip:309@192.168.1.15 SIP/2.0

Via: SIP/2.0/UDP 192.168.1.10;branch=z9hG4bKbda1f3d862585809

From: <sip:308@192.168.1.15>;tag=b96a3ee777b65023

To: <sip:309@192.168.1.15>

Contact: <sip:308@192.168.1.10>

Supported: replaces

Call-ID: 1251b1214bd0ffc0@192.168.1.10

CSeq: 25529 INVITE

User-Agent: Grandstream BT110 1.0.8.33

Max-Forwards: 70

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Content-Type: application/sdp

Content-Length: 225

v=0

o=308 8000 8000 IN IP4 192.168.1.10

s=SIP Call

c=IN IP4 192.168.1.10

t=0 0

m=audio 5004 RTP/AVP 4 18 97

a=sendrecv

a=rtpmap:4 G723/8000

a=rtpmap:18 G729/8000

a=rtpmap:97 iLBC/8000

a=fmtp:97 mode=20

a=ptime:60

{end}

0(18983) [  End of Request (new transaction 3)  ]

0(18983) Adding Record-Route headers

0(18983) Message is relayed; now exiting

0(18983)

[  Reply 100 (Trying) from 192.168.1.11:5060 concerning INVITE  ]

SIP/2.0 100 Trying

Via: SIP/2.0/UDP 192.168.1.15;branch=z9hG4bK9e4e.dbce68c4.0

Via: SIP/2.0/UDP 192.168.1.10;branch=z9hG4bKbda1f3d862585809

From: <sip:308@192.168.1.15>;tag=b96a3ee777b65023

To: <sip:309@192.168.1.15>

Call-ID: 1251b1214bd0ffc0@192.168.1.10

CSeq: 25529 INVITE

User-Agent: Grandstream BT110 1.0.8.33

Content-Length: 0

[  End of Reply (transaction 3)  ]

0(18983)

[  Reply 180 (Ringing) from 192.168.1.11:5060 concerning INVITE  ]

SIP/2.0 180 Ringing

Via: SIP/2.0/UDP 192.168.1.15;branch=z9hG4bK9e4e.dbce68c4.0

Via: SIP/2.0/UDP 192.168.1.10;branch=z9hG4bKbda1f3d862585809

Record-Route: <sip:192.168.1.15;lr=on;ftag=b96a3ee777b65023>

From: <sip:308@192.168.1.15>;tag=b96a3ee777b65023

To: <sip:309@192.168.1.15>;tag=95ff5088b6cf94f7

Call-ID: 1251b1214bd0ffc0@192.168.1.10

CSeq: 25529 INVITE

User-Agent: Grandstream BT110 1.0.8.33

Content-Length: 0

[  End of Reply (transaction 3)  ]

0(18983)

[  Reply 200 (OK) from 192.168.1.11:5060 concerning INVITE  ]

SIP/2.0 200 OK

Via: SIP/2.0/UDP 192.168.1.15;branch=z9hG4bK9e4e.dbce68c4.0

Via: SIP/2.0/UDP 192.168.1.10;branch=z9hG4bKbda1f3d862585809

Record-Route: <sip:192.168.1.15;lr=on;ftag=b96a3ee777b65023>

From: <sip:308@192.168.1.15>;tag=b96a3ee777b65023

To: <sip:309@192.168.1.15>;tag=95ff5088b6cf94f7

Call-ID: 1251b1214bd0ffc0@192.168.1.10

CSeq: 25529 INVITE

User-Agent: Grandstream BT110 1.0.8.33

Contact: <sip:309@192.168.1.11>

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Content-Type: application/sdp

Supported: replaces

Content-Length: 154

v=0

o=309 8000 8000 IN IP4 192.168.1.11

s=SIP Call

c=IN IP4 192.168.1.11

t=0 0

m=audio 5004 RTP/AVP 4

a=sendrecv

a=rtpmap:4 G723/8000

a=ptime:60

{end}

[  End of Reply (transaction 3)  ]

0(18983) [ New Transaction ]

0(18983) [ New Transaction ]

0(18983) [  Method ACK from 192.168.1.10:5060  ]

ACK sip:309@192.168.1.11 SIP/2.0

Via: SIP/2.0/UDP 192.168.1.10;branch=z9hG4bKa2efcaf288b64e74

Route: <sip:192.168.1.15;lr=on;ftag=b96a3ee777b65023>

From: <sip:308@192.168.1.15>;tag=b96a3ee777b65023

To: <sip:309@192.168.1.15>;tag=95ff5088b6cf94f7

Contact: <sip:308@192.168.1.10>

Call-ID: 1251b1214bd0ffc0@192.168.1.10

CSeq: 25529 ACK

User-Agent: Grandstream BT110 1.0.8.33

Max-Forwards: 70

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Content-Length: 0

0(18983) [  End of Request (new transaction 4)  ]

0(18983) Loose Route

0(18983) Message is relayed; now exiting

0(18983) [ New Transaction ]

0(18983) [ New Transaction ]

0(18983) [ New Transaction ]

0(18983) [  Method BYE from 192.168.1.11:5060  ]

BYE sip:308@192.168.1.10 SIP/2.0

Via: SIP/2.0/UDP 192.168.1.11;branch=z9hG4bKeb64fa3b52b9129d

Route: <sip:192.168.1.15;lr=on;ftag=b96a3ee777b65023>

From: <sip:309@192.168.1.15>;tag=95ff5088b6cf94f7

To: <sip:308@192.168.1.15>;tag=b96a3ee777b65023

Supported: replaces

Call-ID: 1251b1214bd0ffc0@192.168.1.10

CSeq: 8746 BYE

User-Agent: Grandstream BT110 1.0.8.33

Max-Forwards: 70

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Content-Length: 0

0(18983) [  End of Request (new transaction 5)  ]

0(18983) Loose Route

0(18983) Message is relayed; now exiting

0(18983)

[  Reply 200 (OK) from 192.168.1.10:5060 concerning BYE  ]

SIP/2.0 200 OK

Via: SIP/2.0/UDP 192.168.1.15;branch=z9hG4bK691d.8ba88547.0

Via: SIP/2.0/UDP 192.168.1.11;branch=z9hG4bKeb64fa3b52b9129d

From: <sip:309@192.168.1.15>;tag=95ff5088b6cf94f7

To: <sip:308@192.168.1.15>;tag=b96a3ee777b65023

Call-ID: 1251b1214bd0ffc0@192.168.1.10

CSeq: 8746 BYE

User-Agent: Grandstream BT110 1.0.8.33

Contact: <sip:308@192.168.1.10>

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Supported: replaces

Content-Length: 0

[  End of Reply (transaction 5)  ]

1(18984) INFO: signal 2 received

0(18983) child process 18984 exited normally, status=0

0(18983) INFO: dont_fork turned on, living on

0(18983) INFO: signal 2 received

0(18983) INFO:mi_fifo:mi_destroy: seems that fifo child is already dead!

0(18983) Thank you for flying openser

SER02:/home/emin#

SER02:/home/emin#

SER02:/home/emin#