SER02:/home/emin/Desktop/trans#

SER02:/home/emin/Desktop/trans#

SER02:/home/emin/Desktop/trans# openser -f ser16.cfg

Listening on

udp: 192.168.1.15 [192.168.1.15]:5060

tcp: 192.168.1.15 [192.168.1.15]:5060

Aliases:

tcp: SER02:5060

udp: SER02:5060

WARNING: no fork mode

0(2606) init_tcp: using epoll_lt as the io watch method (auto detected)

0(0) INFO: statistics manager successfully initialized

0(0) StateLess module - initializing

0(0) TM - initializing...

0(0) Maxfwd module- initializing

0(0) INFO:ul_init_locks: locks array size 512

0(0) TextOPS - initializing

0(0) INFO: udp_init: SO_RCVBUF is initially 109568

0(0) INFO: udp_init: SO_RCVBUF is finally 219136

0(2606) INFO:mi_fifo:mi_child_init(1): extra fifo listener processes created

0(2606) route{...}

0(2606) new transaction or lost flags ...

0(2606)   [ REGISTER from 192.168.1.10:5060 ]

REGISTER sip:192.168.1.15 SIP/2.0

Via: SIP/2.0/UDP 192.168.1.10;branch=z9hG4bK6e90e945988ddf9c

From: <sip:10@192.168.1.15>;tag=b6948c8266ca318e

To: <sip:10@192.168.1.15>

Contact: <sip:10@192.168.1.10>

Supported: replaces

Call-ID: 63b86fbc70ebec8c@192.168.1.10

CSeq: 1749 REGISTER

Expires: 150

User-Agent: Grandstream BT110 1.0.8.33

Max-Forwards: 70

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Content-Length: 0

0(2606)   [ End of REGISTER flagged 1 ]

0(2606) route{...}

0(2606) new transaction or lost flags ...

0(2606)   [ REGISTER from 192.168.1.15:5060 ]

REGISTER sip:192.168.1.15 SIP/2.0

Via: SIP/2.0/UDP 192.168.1.15;branch=z9hG4bK709c.3a3a4087.0

Via: SIP/2.0/UDP 192.168.1.10;branch=z9hG4bK6e90e945988ddf9c

From: <sip:10@192.168.1.15>;tag=b6948c8266ca318e

To: <sip:10@192.168.1.15>

Contact: <sip:10@192.168.1.10>

Supported: replaces

Call-ID: 63b86fbc70ebec8c@192.168.1.10

CSeq: 1749 REGISTER

Expires: 150

User-Agent: Grandstream BT110 1.0.8.33

Max-Forwards: 69

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Content-Length: 0

P-hint: Self-looped request

0(2606)   [ End of REGISTER flagged 2 ]

0(2606) onreply_route[1] {...}

0(2606) an on-going transaction ...

0(2606)   [ 200 (OK) from 192.168.1.15:5060 concerning REGISTER ]

SIP/2.0 200 OK

Via: SIP/2.0/UDP 192.168.1.15;branch=z9hG4bK709c.3a3a4087.0

Via: SIP/2.0/UDP 192.168.1.10;branch=z9hG4bK6e90e945988ddf9c

From: <sip:10@192.168.1.15>;tag=b6948c8266ca318e

To: <sip:10@192.168.1.15>;tag=4f4b3fc66ce1848604aacd9b43692540.fe8a

Call-ID: 63b86fbc70ebec8c@192.168.1.10

CSeq: 1749 REGISTER

Contact: <sip:10@192.168.1.10>;expires=150

Server: OpenSER (1.2.0-notls (i386/linux))

Content-Length: 0

0(2606)   [ End of 200 flagged 1 ]

0(2606) route{...}

0(2606) new transaction or lost flags ...

0(2606)   [ REGISTER from 192.168.1.11:5060 ]

REGISTER sip:192.168.1.15 SIP/2.0

Via: SIP/2.0/UDP 192.168.1.11;branch=z9hG4bK476669e28c4f343b

From: <sip:11@192.168.1.15>;tag=c2d43ba28dfa6d9e

To: <sip:11@192.168.1.15>

Contact: <sip:11@192.168.1.11>

Supported: replaces

Call-ID: 646f0844af38d26c@192.168.1.11

CSeq: 124 REGISTER

Expires: 150

User-Agent: Grandstream BT110 1.0.8.33

Max-Forwards: 70

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Content-Length: 0

0(2606)   [ End of REGISTER flagged 3 ]

0(2606) route{...}

0(2606) new transaction or lost flags ...

0(2606)   [ REGISTER from 192.168.1.15:5060 ]

REGISTER sip:192.168.1.15 SIP/2.0

Via: SIP/2.0/UDP 192.168.1.15;branch=z9hG4bK7f83.be176f.0

Via: SIP/2.0/UDP 192.168.1.11;branch=z9hG4bK476669e28c4f343b

From: <sip:11@192.168.1.15>;tag=c2d43ba28dfa6d9e

To: <sip:11@192.168.1.15>

Contact: <sip:11@192.168.1.11>

Supported: replaces

Call-ID: 646f0844af38d26c@192.168.1.11

CSeq: 124 REGISTER

Expires: 150

User-Agent: Grandstream BT110 1.0.8.33

Max-Forwards: 69

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Content-Length: 0

P-hint: Self-looped request

0(2606)   [ End of REGISTER flagged 4 ]

0(2606) onreply_route[1] {...}

0(2606) an on-going transaction ...

0(2606)   [ 200 (OK) from 192.168.1.15:5060 concerning REGISTER ]

SIP/2.0 200 OK

Via: SIP/2.0/UDP 192.168.1.15;branch=z9hG4bK7f83.be176f.0

Via: SIP/2.0/UDP 192.168.1.11;branch=z9hG4bK476669e28c4f343b

From: <sip:11@192.168.1.15>;tag=c2d43ba28dfa6d9e

To: <sip:11@192.168.1.15>;tag=4f4b3fc66ce1848604aacd9b43692540.a7dd

Call-ID: 646f0844af38d26c@192.168.1.11

CSeq: 124 REGISTER

Contact: <sip:11@192.168.1.11>;expires=150

Server: OpenSER (1.2.0-notls (i386/linux))

Content-Length: 0

0(2606)   [ End of 200 flagged 3 ]

1(2607) INFO: signal 2 received

1(2607) INFO: signal 15 received

0(2606) INFO: signal 2 received

0(2606) INFO:mi_fifo:mi_destroy: seems that fifo child is already dead!

0(2606) Thank you for flying openser

SER02:/home/emin/Desktop/trans#