SER02:/home/emin#

SER02:/home/emin#

SER02:/home/emin#

SER02:/home/emin#

SER02:/home/emin# openser

Listening on

udp: 192.168.1.15 [192.168.1.15]:5060

tcp: 192.168.1.15 [192.168.1.15]:5060

Aliases:

tcp: SER02:5060

udp: SER02:5060

WARNING: no fork mode

0(25604) init_tcp: using epoll_lt as the io watch method (auto detected)

0(0) INFO: statistics manager successfully initialized

0(0) StateLess module - initializing

0(0) TM - initializing...

0(0) Maxfwd module- initializing

0(0) TextOPS - initializing

0(0) INFO: udp_init: SO_RCVBUF is initially 109568

0(0) INFO: udp_init: SO_RCVBUF is finally 219136

2(25606) INFO: fifo process starting: 25606

2(25606) INFO:open_uac_fifo: fifo server up at /tmp/openser_fifo...

2(25606) INFO: no fifo_db_url given - fifo DB commands disabled!

0(25604)

[  Method REGISTER from 192.168.1.11  ]

REGISTER sip:192.168.1.15 SIP/2.0

Via: SIP/2.0/UDP 192.168.1.11;branch=z9hG4bKaff1d78cd365837c

From: <sip:309@192.168.1.15>;tag=55767c7bdf9cae67

To: <sip:309@192.168.1.15>

Contact: <sip:309@192.168.1.11>

Supported: replaces

Call-ID: 5e18a451b88e1be3@192.168.1.11

CSeq: 102 REGISTER

Expires: 150

User-Agent: Grandstream BT110 1.0.8.33

Max-Forwards: 70

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Content-Length: 0

[  End of Request  ]

0(25604) Location is saved; now exiting

0(25604)

[  Method REGISTER from 192.168.1.10  ]

REGISTER sip:192.168.1.15 SIP/2.0

Via: SIP/2.0/UDP 192.168.1.10;branch=z9hG4bK899135c8a4a8ec49

From: <sip:308@192.168.1.15>;tag=1d94af821dca848e

To: <sip:308@192.168.1.15>

Contact: <sip:308@192.168.1.10>

Supported: replaces

Call-ID: 2eeecc4c27282584@192.168.1.10

CSeq: 102 REGISTER

Expires: 150

User-Agent: Grandstream BT110 1.0.8.33

Max-Forwards: 70

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Content-Length: 0

[  End of Request  ]

0(25604) Location is saved; now exiting

0(25604)

[  Method REGISTER from 192.168.1.11  ]

REGISTER sip:192.168.1.15 SIP/2.0

Via: SIP/2.0/UDP 192.168.1.11;branch=z9hG4bKcc6ad9958ff0e258

From: <sip:309@192.168.1.15>;tag=9183d0336a361f53

To: <sip:309@192.168.1.15>

Contact: <sip:309@192.168.1.11>

Supported: replaces

Call-ID: 93e015602c6124b8@192.168.1.11

CSeq: 100 REGISTER

Expires: 150

User-Agent: Grandstream BT110 1.0.8.33

Max-Forwards: 70

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Content-Length: 0

[  End of Request  ]

0(25604) Location is saved; now exiting

0(25604)

[  Method INVITE from 192.168.1.10  ]

INVITE sip:309@192.168.1.15 SIP/2.0

Via: SIP/2.0/UDP 192.168.1.10;branch=z9hG4bKf3c60221ce03445c

From: <sip:308@192.168.1.15>;tag=fce371520693b722

To: <sip:309@192.168.1.15>

Contact: <sip:308@192.168.1.10>

Supported: replaces

Call-ID: 8c80d06175b1eb80@192.168.1.10

CSeq: 28826 INVITE

User-Agent: Grandstream BT110 1.0.8.33

Max-Forwards: 70

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Content-Type: application/sdp

Content-Length: 225

v=0

o=308 8000 8000 IN IP4 192.168.1.10

s=SIP Call

c=IN IP4 192.168.1.10

t=0 0

m=audio 5004 RTP/AVP 4 18 97

a=sendrecv

a=rtpmap:4 G723/8000

a=rtpmap:18 G729/8000

a=rtpmap:97 iLBC/8000

a=fmtp:97 mode=20

a=ptime:60

{end}

[  End of Request  ]

0(25604) Adding the Route header

0(25604) Message is relayed; now exiting

0(25604)

[  Reply 100 (Trying) from 192.168.1.11 concerning INVITE  ]

SIP/2.0 100 Trying

Via: SIP/2.0/UDP 192.168.1.15;branch=z9hG4bKb992.ec6bce33.0

Via: SIP/2.0/UDP 192.168.1.10;branch=z9hG4bKf3c60221ce03445c

From: <sip:308@192.168.1.15>;tag=fce371520693b722

To: <sip:309@192.168.1.15>

Call-ID: 8c80d06175b1eb80@192.168.1.10

CSeq: 28826 INVITE

User-Agent: Grandstream BT110 1.0.8.33

Content-Length: 0

[  End of Reply  ]

0(25604)

[  Reply 180 (Ringing) from 192.168.1.11 concerning INVITE  ]

SIP/2.0 180 Ringing

Via: SIP/2.0/UDP 192.168.1.15;branch=z9hG4bKb992.ec6bce33.0

Via: SIP/2.0/UDP 192.168.1.10;branch=z9hG4bKf3c60221ce03445c

Record-Route: <sip:192.168.1.15;lr=on;ftag=fce371520693b722>

From: <sip:308@192.168.1.15>;tag=fce371520693b722

To: <sip:309@192.168.1.15>;tag=352ec99f5ffd23cf

Call-ID: 8c80d06175b1eb80@192.168.1.10

CSeq: 28826 INVITE

User-Agent: Grandstream BT110 1.0.8.33

Content-Length: 0

[  End of Reply  ]

0(25604)

[  Reply 200 (OK) from 192.168.1.11 concerning INVITE  ]

SIP/2.0 200 OK

Via: SIP/2.0/UDP 192.168.1.15;branch=z9hG4bKb992.ec6bce33.0

Via: SIP/2.0/UDP 192.168.1.10;branch=z9hG4bKf3c60221ce03445c

Record-Route: <sip:192.168.1.15;lr=on;ftag=fce371520693b722>

From: <sip:308@192.168.1.15>;tag=fce371520693b722

To: <sip:309@192.168.1.15>;tag=352ec99f5ffd23cf

Call-ID: 8c80d06175b1eb80@192.168.1.10

CSeq: 28826 INVITE

User-Agent: Grandstream BT110 1.0.8.33

Contact: <sip:309@192.168.1.11>

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Content-Type: application/sdp

Supported: replaces

Content-Length: 154

v=0

o=309 8000 8000 IN IP4 192.168.1.11

s=SIP Call

c=IN IP4 192.168.1.11

t=0 0

m=audio 5004 RTP/AVP 4

a=sendrecv

a=rtpmap:4 G723/8000

a=ptime:60

[  End of Reply  ]

0(25604)

[  Method ACK from 192.168.1.10  ]

ACK sip:309@192.168.1.11 SIP/2.0

Via: SIP/2.0/UDP 192.168.1.10;branch=z9hG4bK9678e44f4ea235df

Route: <sip:192.168.1.15;lr=on;ftag=fce371520693b722>

From: <sip:308@192.168.1.15>;tag=fce371520693b722

To: <sip:309@192.168.1.15>;tag=352ec99f5ffd23cf

Contact: <sip:308@192.168.1.10>

Call-ID: 8c80d06175b1eb80@192.168.1.10

CSeq: 28826 ACK

User-Agent: Grandstream BT110 1.0.8.33

Max-Forwards: 70

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Content-Length: 0

[  End of Request  ]

0(25604) Adding the Route header

0(25604) Loose Route

0(25604) Message is relayed; now exiting

0(25604)

[  Method REGISTER from 192.168.1.10  ]

REGISTER sip:192.168.1.15 SIP/2.0

Via: SIP/2.0/UDP 192.168.1.10;branch=z9hG4bK46df157b527d825b

From: <sip:308@192.168.1.15>;tag=d7ec4791911c6e44

To: <sip:308@192.168.1.15>

Contact: <sip:308@192.168.1.10>

Supported: replaces

Call-ID: ec1234aabde0e4e8@192.168.1.10

CSeq: 100 REGISTER

Expires: 150

User-Agent: Grandstream BT110 1.0.8.33

Max-Forwards: 70

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Content-Length: 0

[  End of Request  ]

0(25604) Location is saved; now exiting

0(25604)

[  Method INFO from 192.168.1.10  ]

INFO sip:309@192.168.1.11 SIP/2.0

Via: SIP/2.0/UDP 192.168.1.10;branch=z9hG4bK93bc61c18d96eb8b

Route: <sip:192.168.1.15;lr=on;ftag=fce371520693b722>

From: <sip:308@192.168.1.15>;tag=fce371520693b722

To: <sip:309@192.168.1.15>;tag=352ec99f5ffd23cf

Contact: <sip:308@192.168.1.10>

Supported: replaces

Call-ID: 8c80d06175b1eb80@192.168.1.10

CSeq: 28827 INFO

User-Agent: Grandstream BT110 1.0.8.33

Max-Forwards: 70

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Content-Type: application/dtmf-relay

Content-Length: 23

Signal=2

Duration=4800{end}

[  End of Request  ]

0(25604) Adding the Route header

0(25604) Loose Route

0(25604) Message is relayed; now exiting

0(25604)

[  Reply 200 (OK) from 192.168.1.11 concerning INFO  ]

SIP/2.0 200 OK

Via: SIP/2.0/UDP 192.168.1.15;branch=z9hG4bKc992.e100efc6.0

Via: SIP/2.0/UDP 192.168.1.10;branch=z9hG4bK93bc61c18d96eb8b

Record-Route: <sip:192.168.1.15;lr=on;ftag=fce371520693b722>

From: <sip:308@192.168.1.15>;tag=fce371520693b722

To: <sip:309@192.168.1.15>;tag=352ec99f5ffd23cf

Call-ID: 8c80d06175b1eb80@192.168.1.10

CSeq: 28827 INFO

User-Agent: Grandstream BT110 1.0.8.33

Contact: <sip:309@192.168.1.11>

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Supported: replaces

Content-Length: 0

[  End of Reply  ]

0(25604)

[  Method INFO from 192.168.1.11  ]

INFO sip:308@192.168.1.10 SIP/2.0

Via: SIP/2.0/UDP 192.168.1.11;branch=z9hG4bKd51a30c261b5691e

Route: <sip:192.168.1.15;lr=on;ftag=fce371520693b722>

From: <sip:309@192.168.1.15>;tag=352ec99f5ffd23cf

To: <sip:308@192.168.1.15>;tag=fce371520693b722

Contact: <sip:309@192.168.1.11>

Supported: replaces

Call-ID: 8c80d06175b1eb80@192.168.1.10

CSeq: 52519 INFO

User-Agent: Grandstream BT110 1.0.8.33

Max-Forwards: 70

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Content-Type: application/dtmf-relay

Content-Length: 22

Signal=3

Duration=480{end}

[  End of Request  ]

0(25604) Adding the Route header

0(25604) Loose Route

0(25604) Message is relayed; now exiting

0(25604)

[  Reply 200 (OK) from 192.168.1.10 concerning INFO  ]

SIP/2.0 200 OK

Via: SIP/2.0/UDP 192.168.1.15;branch=z9hG4bKb593.5d620aa5.0

Via: SIP/2.0/UDP 192.168.1.11;branch=z9hG4bKd51a30c261b5691e

Record-Route: <sip:192.168.1.15;lr=on;ftag=352ec99f5ffd23cf>

From: <sip:309@192.168.1.15>;tag=352ec99f5ffd23cf

To: <sip:308@192.168.1.15>;tag=fce371520693b722

Call-ID: 8c80d06175b1eb80@192.168.1.10

CSeq: 52519 INFO

User-Agent: Grandstream BT110 1.0.8.33

Contact: <sip:308@192.168.1.10>

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Supported: replaces

Content-Length: 0

[  End of Reply  ]

0(25604)

[  Method BYE from 192.168.1.11  ]

BYE sip:308@192.168.1.10 SIP/2.0

Via: SIP/2.0/UDP 192.168.1.11;branch=z9hG4bKd0b04fbb081eb9ab

Route: <sip:192.168.1.15;lr=on;ftag=352ec99f5ffd23cf>

From: <sip:309@192.168.1.15>;tag=352ec99f5ffd23cf

To: <sip:308@192.168.1.15>;tag=fce371520693b722

Supported: replaces

Call-ID: 8c80d06175b1eb80@192.168.1.10

CSeq: 52520 BYE

User-Agent: Grandstream BT110 1.0.8.33

Max-Forwards: 70

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Content-Length: 0

[  End of Request  ]

0(25604) Adding the Route header

0(25604) Loose Route

0(25604) Message is relayed; now exiting

0(25604)

[  Reply 200 (OK) from 192.168.1.10 concerning BYE  ]

SIP/2.0 200 OK

Via: SIP/2.0/UDP 192.168.1.15;branch=z9hG4bK0593.583e3506.0

Via: SIP/2.0/UDP 192.168.1.11;branch=z9hG4bKd0b04fbb081eb9ab

Record-Route: <sip:192.168.1.15;lr=on;ftag=352ec99f5ffd23cf>

From: <sip:309@192.168.1.15>;tag=352ec99f5ffd23cf

To: <sip:308@192.168.1.15>;tag=fce371520693b722

Call-ID: 8c80d06175b1eb80@192.168.1.10

CSeq: 52520 BYE

User-Agent: Grandstream BT110 1.0.8.33

Contact: <sip:308@192.168.1.10>

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Supported: replaces

Content-Length: 0

[  End of Reply  ]

0(25604)

[  Method REGISTER from 192.168.1.11  ]

REGISTER sip:192.168.1.15 SIP/2.0

Via: SIP/2.0/UDP 192.168.1.11;branch=z9hG4bK927ab0c8fb4a0ae8

From: <sip:309@192.168.1.15>;tag=9183d0336a361f53

To: <sip:309@192.168.1.15>

Contact: <sip:309@192.168.1.11>

Supported: replaces

Call-ID: 93e015602c6124b8@192.168.1.11

CSeq: 101 REGISTER

Expires: 150

User-Agent: Grandstream BT110 1.0.8.33

Max-Forwards: 70

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Content-Length: 0

[  End of Request  ]

0(25604) Location is saved; now exiting

0(25604)

[  Method REGISTER from 192.168.1.10  ]

REGISTER sip:192.168.1.15 SIP/2.0

Via: SIP/2.0/UDP 192.168.1.10;branch=z9hG4bK650ac98820e98678

From: <sip:308@192.168.1.15>;tag=d7ec4791911c6e44

To: <sip:308@192.168.1.15>

Contact: <sip:308@192.168.1.10>

Supported: replaces

Call-ID: ec1234aabde0e4e8@192.168.1.10

CSeq: 101 REGISTER

Expires: 150

User-Agent: Grandstream BT110 1.0.8.33

Max-Forwards: 70

Allow: INVITE,ACK,CANCEL,BYE,NOTIFY,REFER,OPTIONS,INFO,SUBSCRIBE

Content-Length: 0

[  End of Request  ]

0(25604) Location is saved; now exiting

1(25605) INFO: signal 2 received

0(25604) child process 25605 exited normally, status=0

0(25604) INFO: dont_fork turned on, living on

2(25606) INFO: signal 15 received

0(25604) Thank you for flying openser

SER02:/home/emin#

SER02:/home/emin#

SER02:/home/emin#

SER02:/home/emin#

SER02:/home/emin#